Purpose: Universo commissioning

| Version | Date | Author | Comment |
| --- | --- | --- | --- |
| A | 19/06/2018 | PY BALME | Creation |

This document provides the information needed to study the Universo column IT/IP architecture.

Contents:

- Video surveillance module

- Sound system module

  - Analogue sound system

  - 100V sound system

  - IP Audio

- WiFi module

- Modem router module

- POE switch

# Video surveillance module

The camera module in the Universo column is supplied with a Samsung XNB-6001 box placed inside the enclosure for IP connection and power supply to the camera and lens. The camera and lens are located in the module behind the polycarbonate bowl.

The box and camera specifications are available in the document below:

https://www.hanwhasecurity.com/media/attachment/file/d/a/datasheet_xnb-6001_sla- t2480_sla-t4680_sla-t1080f_180104_edit_1.pdf

On first power up, the camera address can be found using the IP installer.exe software, which can be downloaded using the link below:

https://www.hanwhasecurity.com/wisenet-ip-installer/

The default login and password are: Login : admin

Password: admin

Follow the instructions in the document linked below to configure the camera:

https://www.eos.com.au/pub/media/doc/wisenet/Manuals_XNB-6001_171206_EN.pdf

The XNB-6001 camera is compatible with the open ONVIF platform and can therefore be integrated into an existing video surveillance system.

# Sound system module

  - Analogue sound system

The column is equipped with a module including a loudspeaker with an RMS power of 15W and an impedance of 8 Ohm.

The sound system cable must be connected to an audio amplifier output.

  - 100V sound system

The column is fitted with a module with a 100V transformer and a 15W loudspeaker with an impedance of 8 Ohm.

The sound system cable must be connected to the 100V output of a transformer connected to an audio amplifier.

  - IP Audio

The column is equipped with a loudspeaker module and a decoder/amplifier. Customers will also have access to an encoder that allows transmission of an audio source to the column from a control station.

The default IP address for the various modules is 192.168.10.159.

A presentation of the street sound system application is available using the link below: http://www.ateis.com/content/_documents/application-sono-rue.pdf

The decoder/amplifier specifications are available using the link below:
http://www.ateis.com/content/_documents/FT FR_TERRA_EXA_V4.0.pdf

The encoder specifications are available using the link below:
http://www.ateis.com/content/_documents/FT FR_TERRA_IEX_V3.0.pdf

The document below presents the software part used to configure the IP audio system:

http://www.ateis.com/content/_documents/VOX@ Manuel utilisateur 1.3_TERRA

-EXA.pdf

If the console option has been ordered, the product specifications are described in the document using the link below: http://www.ateis.com/content/_documents/FT FR_VOXA-D-V2.0.pdf

# Wifi module

The column is fitted with a cover that has a built in wifi module and antenna. The module's default IP address is 192.168.88.1

The username is ''admin'' and there is no password

The wifi antenna specifications are available using the link below:
https://i.mt.lv/routerboard/files/wAP_ac-170927121818.pdf

The configuration documentation for the antenna using the integrated software is available using the link below:

https://wiki.mikrotik.com/wiki/Manual:TOC

# Modem/router module

The column is fitted with a cover containing a router and a GPRS/WIFI antenna.

This option makes it possible to have a network connected to the internet and to share it using wifi. The Internet connection will use a SIM card not supplied by Eclatec.

The quick start guide is available using the link below: https://cradlepoint.com/sites/default/files/upload-file/1100-qsg-full-082017.pdf

The router technical data sheet is available using the link below: https://cradlepoint.com/sites/default/files/upload- file/cradlepoint_ibr1100_spec_sheet_3.pdf

The router configuration manual can be found using the link below: https://cradlepoint.com/sites/default/files/upload-file/cradlepoint_ibr1100_manual_4.pdf

The antenna technical data sheet can be found in the document below: http://www.panorama- antennas.com/site/index.php?route=product/product&product_id=70&search=MiMo&descr iption=1

# POE switch

The POE switch is used when several IP devices need to be connected to the column network.

The switch technical data sheet is available using the link below: https://www.planet.com.tw/storage/products/32358/C-ISW-504PS_s.pdf

The user manual is available using the link below: https://www.planet.com.tw/storage/products/32358/EM-ISW5xxPS Series_1.0.pdf